Instillation Instructions

- Remove clutch cable from clutch arm (on engine), and clutch cable and sheath from cable guide, located in the engine valley.

- If you need to relocate the clutch arm, now is the time to do it. Remove the nut, pry arm up and off the clutch shaft. The shaft and arm are splined. I rotate the clutch arm so it is parallel with the chain. This will make for a more secure connection at the arm and cable.

- Remove clutch cable from engine.

- Remove cable itself, clean and grease.

- Discard small spring; use large spring for heat shield at the cylinder.

- Unscrew the cable guide from the center post. See “POST” picture below.The post can be reinstalled at this time

- Remove the 10mm bolt from the bracket and install the cable guide

- Thread the 10mm bolt (supplied) into hole and secure into the hole in the clutch cover that you removed the screw from.

- Tighten the bolt only enough to hold it in place do not tighten fully at this time.

- .Check for proper cable and housing length as the housing might need to be shortened

- Route the cable through the cable guide and around roller. Secure the cable in the clutch arm.

- Tighten the cable (no need for clutch adjustment at this time), let the bracket line itself up.

- Tighten the bracket bolt (supplied). The bracket is now in place.

- Adjust clutch cable as you would do normally.

This bracket comes with a 100% satisfaction, no questions asked warranty.

Thank you for your purchase.

Ronald V. Becker

85 Southern Hills Drive

Calera, Alabama 35040

(205) 668-3231 Home

(205) 602-5578 Cell