- Kiểm tra mực dầu.

- Thân chào các bạn mới, dưới đây là một bài dịch nhỏ mình đã trích ra từ một bài dịch lớn để gởi cho các bạn mong các bạn làm nó trước khi trở thành thành viên chính thức của ECAE. Các bạn hãy làm nó và gởi lại vào ngày 13/08/2011 (thứ 7). Bài của các bạn, các bạn hãy gởi lại cho mình theo địa chỉ duyleauto@oto-hui.com

- Ngày giao: 08/10/2011

- Phần dịch màu đỏ là của: tanbka

- Phần dịch màu xanh là của: nhuminh

- Check Fluid Levels

A car operates using a variety of fluids, engine oil, either standard or automatic transmission fluid, power steering fluid, brake fluid, front and rear differential fluids on some models. All fluids must be maintained at a proper level, while some are easily checked others will require a lift. First, start with the vehicle in park on level ground, in cool condition with the engine off. Next, release the hood latch and lift the hood, the hood will only open slightly because all hoods have been designed with a secondary safety latch that must be activated before the hood will open completely.

Checking Engine Oil Level - First check the engine oil level (if you are changing the engine oil, check the oil level after the oil change has been performed) This engine oil level dip stick is usually indicated by a yellow handle. There are two level indicator marks on the dip stick, upper level and lower level. The distance in between these marks represents one quart, if the oil is in between the marks the level is okay. If the oil level is below the lower level mark or not on the dip stick at all engine oil must be added until oil is between the inductor lines. Use a shop towel is wipe the excess oil from the dip stick and random drips.

Checking Engine Oil

Engine Oil Dip Stick Level Indicator

Engine Oil Fill Port